/*

* SPLC780D LCD Driver - Structured Code

* PA2 = EN (Enable)

* PA3 = RS (Register Select)

* PB0-PB7 = DB0-DB7 (8-bit data)

* PE0-PE3 = Keypad rows

* PD0-PD3 = Keypad columns

*/

#include "tm4c123gh6pm.h"

// System Control Registers

#define SYSCTL_RCGC2_R      (*((volatile unsigned long *)0x400FE108))

#define SYSCTL_RCC_R        (*((volatile unsigned long *)0x400FE060))

#define SYSCTL_RCC2_R       (*((volatile unsigned long *)0x400FE070))

#define SYSCTL_RIS_R        (*((volatile unsigned long *)0x400FE050))

// SysTick Registers

#define NVIC_ST_CTRL_R      (*((volatile unsigned long *)0xE000E010))

#define NVIC_ST_RELOAD_R    (*((volatile unsigned long *)0xE000E014))

#define NVIC_ST_CURRENT_R   (*((volatile unsigned long *)0xE000E018))

// Port A Registers

#define GPIO_PORTA_LOCK_R   (*((volatile unsigned long *)0x40004520))

#define GPIO_PORTA_CR_R     (*((volatile unsigned long *)0x40004524))

#define GPIO_PORTA_AMSEL_R  (*((volatile unsigned long *)0x40004528))

#define GPIO_PORTA_DIR_R    (*((volatile unsigned long *)0x40004400))

#define GPIO_PORTA_DEN_R    (*((volatile unsigned long *)0x4000451C))

#define GPIO_PORTA_AFSEL_R  (*((volatile unsigned long *)0x40004420))

#define GPIO_PORTA_PCTL_R   (*((volatile unsigned long *)0x4000452C))

#define GPIO_PORTA_DATA_R   (*((volatile unsigned long *)0x400043FC))

// Port B Registers

#define GPIO_PORTB_LOCK_R   (*((volatile unsigned long *)0x40005520))

#define GPIO_PORTB_CR_R     (*((volatile unsigned long *)0x40005524))

#define GPIO_PORTB_AMSEL_R  (*((volatile unsigned long *)0x40005528))

#define GPIO_PORTB_DIR_R    (*((volatile unsigned long *)0x40005400))

#define GPIO_PORTB_DEN_R    (*((volatile unsigned long *)0x4000551C))

#define GPIO_PORTB_AFSEL_R  (*((volatile unsigned long *)0x40005420))

#define GPIO_PORTB_PCTL_R   (*((volatile unsigned long *)0x4000552C))

#define GPIO_PORTB_DATA_R   (*((volatile unsigned long *)0x400053FC))

// Port D Registers

#define GPIO_PORTD_LOCK_R   (*((volatile unsigned long *)0x40007520))

#define GPIO_PORTD_CR_R     (*((volatile unsigned long *)0x40007524))

#define GPIO_PORTD_AMSEL_R  (*((volatile unsigned long *)0x40007528))

#define GPIO_PORTD_DIR_R    (*((volatile unsigned long *)0x40007400))

#define GPIO_PORTD_DEN_R    (*((volatile unsigned long *)0x4000751C))

#define GPIO_PORTD_AFSEL_R  (*((volatile unsigned long *)0x40007420))

#define GPIO_PORTD_PCTL_R   (*((volatile unsigned long *)0x4000752C))

#define GPIO_PORTD_PUR_R    (*((volatile unsigned long *)0x40007510))

#define GPIO_PORTD_DATA_R   (*((volatile unsigned long *)0x400073FC))

// Port E Registers

#define GPIO_PORTE_LOCK_R   (*((volatile unsigned long *)0x40024520))

#define GPIO_PORTE_CR_R     (*((volatile unsigned long *)0x40024524))

#define GPIO_PORTE_AMSEL_R  (*((volatile unsigned long *)0x40024528))

#define GPIO_PORTE_DIR_R    (*((volatile unsigned long *)0x40024400))

#define GPIO_PORTE_DEN_R    (*((volatile unsigned long *)0x4002451C))

#define GPIO_PORTE_AFSEL_R  (*((volatile unsigned long *)0x40024420))

#define GPIO_PORTE_PCTL_R   (*((volatile unsigned long *)0x4002452C))

#define GPIO_PORTE_PUR_R    (*((volatile unsigned long *)0x40024510))

#define GPIO_PORTE_DATA_R   (*((volatile unsigned long *)0x400243FC))

// Pin definitions

#define LCD_EN      0x04    // PA2

#define LCD_RS      0x08    // PA3

// Keypad definitions

char keypad_chars[4][4] = {

{'1', '2', '3', 'A'},

{'4', '5', '6', 'B'},

{'7', '8', '9', 'C'},

{'*', '0', '#', 'D'}

};

// Function prototypes

void InitializePorts(void);

void InitializeSysTick(void);

void InitializeLCD(void);

void LCD_Enable(void);

void LCD_SendNibble(unsigned char nibble, unsigned char rs);

void LCD_SendByte(unsigned char data, unsigned char rs);

void InitializeKeypad(void);

void SysTick_Debounce(void);

char ReadKeypad(void);

char DecodeKeypress(unsigned char row, unsigned char col);

//============================================================================

// Initialize all ports

//============================================================================

void InitializePorts(void) {

volatile unsigned long delay;

// Enable ports A, B, D, E

SYSCTL_RCGC2_R |= 0x1B;  // Ports A(0), B(1), D(3), E(4)

delay = SYSCTL_RCGC2_R;

// Port A setup (LCD control: PA2=EN, PA3=RS)

GPIO_PORTA_LOCK_R = 0x4C4F434B; // Unlocks Port A for configuration

GPIO_PORTA_CR_R = 0xFF; // Commits all 8 pins for modification

GPIO_PORTA_AMSEL_R = 0x00;

GPIO_PORTA_PCTL_R = 0x00000000;

GPIO_PORTA_DIR_R |= (LCD_EN | LCD_RS); // Sets PA2 and PA3 as outputs

GPIO_PORTA_AFSEL_R = 0x00;

GPIO_PORTA_DEN_R |= (LCD_EN | LCD_RS);

// Port B setup (LCD data: PB0-PB7 = DB0-DB7)

GPIO_PORTB_LOCK_R = 0x4C4F434B; // Unlocks Port B for configuration

GPIO_PORTB_CR_R = 0xFF; // Commits all 8 pins for modification

GPIO_PORTB_AMSEL_R = 0x00;

GPIO_PORTB_PCTL_R = 0x00000000;

GPIO_PORTB_DIR_R = 0xFF;  // Sets all pins as outputs

GPIO_PORTB_AFSEL_R = 0x00;

GPIO_PORTB_DEN_R = 0xFF;  // Digitally enable all pins

// Port D setup (keypad columns: PD0-PD3)

GPIO_PORTD_LOCK_R = 0x4C4F434B;

GPIO_PORTD_CR_R = 0xFF;

GPIO_PORTD_AMSEL_R = 0x00;

GPIO_PORTD_PCTL_R = 0x00000000;

GPIO_PORTD_DIR_R &= ~0x0F; // PD0-3 inputs

GPIO_PORTD_AFSEL_R = 0x00;

GPIO_PORTD_PUR_R |= 0x0F;  // Pull-ups on PD0-3

GPIO_PORTD_DEN_R |= 0x0F;

// Port E setup (keypad rows: PE0-PE3)

GPIO_PORTE_LOCK_R = 0x4C4F434B;

GPIO_PORTE_CR_R = 0x3F;

GPIO_PORTE_AMSEL_R = 0x00;

GPIO_PORTE_PCTL_R = 0x00000000;

GPIO_PORTE_DIR_R |= 0x0F; // PE0-3 outputs

GPIO_PORTE_AFSEL_R = 0x00;

GPIO_PORTE_DEN_R |= 0x0F;

// Initialize pin states

GPIO_PORTA_DATA_R &= ~(LCD_EN | LCD_RS);

GPIO_PORTB_DATA_R = 0x00;

GPIO_PORTE_DATA_R |= 0x0F; // All keypad rows high

}

//============================================================================

// Initialize SysTick

//============================================================================

void InitializeSysTick(void) {

NVIC_ST_CTRL_R = 0;               // Disable SysTick

NVIC_ST_RELOAD_R = 0x00FFFFFF;    // Max reload value

NVIC_ST_CURRENT_R = 0;            // Clear current

NVIC_ST_CTRL_R = 0x00000005;      // Enable with core clock

}

//============================================================================

// Initialize LCD

//============================================================================

void InitializeLCD(void) {

// Wait 62.5 ms for LCD to power up

// 1,000,000 cycles at 16 MHz = 62.5 ms

// Power-on delay

for(volatile int i = 0; i < 1000000; i++);

// Ensure that EN and RS are low before starting

// Initialize control lines

GPIO_PORTA_DATA_R &= ~(LCD_EN | LCD_RS); // Initalize LCD Control Pins

// 8-bit initialization sequence

LCD_SendByte(0x30, 0);  // Function set: 8-bit

for(volatile int i = 0; i < 500000; i++); // 5ms delay

LCD_SendByte(0x30, 0);  // Function set: 8-bit

for(volatile int i = 0; i < 15000; i++);  // 150us delay

LCD_SendByte(0x30, 0);  // Function set: 8-bit

for(volatile int i = 0; i < 15000; i++);  // 150us delay

// Configure LCD settings

// 0x38 = 0011 1000

// Bit 4: DL = 1 (8-bit data length)

// Bit 3: N = 1 (2 display lines)

// Bit 2: F = 0 (5x8 character font)

LCD_SendByte(0x38, 0);  // 8-bit, 2 lines, 5x8 font

LCD_SendByte(0x08, 0);  // Display off

LCD_SendByte(0x01, 0);  // Clear display

for(volatile int i = 0; i < 200000; i++); // Clear needs extra time

LCD_SendByte(0x06, 0);  // Entry mode: increment

LCD_SendByte(0x0C, 0);  // Display on, cursor off

}

//============================================================================

// EN function for enabling the LCD

//============================================================================

void LCD_Enable(void) {

GPIO_PORTA_DATA_R |= LCD_EN;    // EN high

for(volatile int i = 0; i < 100; i++);  // Short delay

GPIO_PORTA_DATA_R &= ~LCD_EN;   // EN low

for(volatile int i = 0; i < 5000; i++); // Command delay

}

//============================================================================

// Nibble send (use the RS on the LCD) - Note: Using full byte for 8-bit mode

//============================================================================

void LCD_SendNibble(unsigned char nibble, unsigned char rs) {

// Set RS line

if(rs) {

GPIO_PORTA_DATA_R |= LCD_RS;    // RS = 1 (data mode)

} else {

GPIO_PORTA_DATA_R &= ~LCD_RS;   // RS = 0 (command mode)

}

// Put nibble on upper 4 bits of Port B (for compatibility)

GPIO_PORTB_DATA_R = (GPIO_PORTB_DATA_R & 0x0F) | (nibble & 0xF0);

LCD_Enable();

}

//============================================================================

// Byte send

//============================================================================

void LCD_SendByte(unsigned char data, unsigned char rs) {

// Set RS line

if(rs) {

GPIO_PORTA_DATA_R |= LCD_RS;    // RS = 1 (data mode)

} else {

GPIO_PORTA_DATA_R &= ~LCD_RS;   // RS = 0 (command mode)

}

// Put full byte on Port B

GPIO_PORTB_DATA_R = data;

LCD_Enable();

}

//============================================================================

// Initialize keypad

//============================================================================

void InitializeKeypad(void) {

// Keypad is already initialized in InitializePorts()

// This function can be used for any additional keypad setup

GPIO_PORTE_DATA_R |= 0x0F; // Ensure all rows are high

}

//============================================================================

// SysTick for debouncing

//============================================================================

void SysTick_Debounce(void) {

NVIC_ST_RELOAD_R = 500000 - 1;  // ~10ms at 50MHz

NVIC_ST_CURRENT_R = 0;

while((NVIC_ST_CTRL_R & 0x00010000) == 0){}; // Wait for flag

}

//============================================================================

// Read keypad

//============================================================================

char ReadKeypad(void) {

for(unsigned char row = 0; row < 4; row++) {

// Set all rows high

GPIO_PORTE_DATA_R |= 0x0F;

// Set current row low

GPIO_PORTE_DATA_R &= ~(1 << row);

// Small delay for signal to settle

for(volatile int i = 0; i < 1000; i++);

// Read columns

unsigned char cols = GPIO_PORTD_DATA_R & 0x0F;

// Check each column

for(unsigned char col = 0; col < 4; col++) {

if(!(cols & (1 << col))) { // Column is low (key pressed)

// Reset all rows high

GPIO_PORTE_DATA_R |= 0x0F;

return DecodeKeypress(row, col);

}

}

}

// No key pressed - set all rows high

GPIO_PORTE_DATA_R |= 0x0F;

return 0;

}

//============================================================================

// Decode keypress

//============================================================================

char DecodeKeypress(unsigned char row, unsigned char col) {

if(row < 4 && col < 4) {

return keypad_chars[row][col];

}

return 0; // Invalid key

}

//============================================================================

// Main function

//============================================================================

int main(void) {

// System initialization

InitializePorts();

InitializeSysTick();

InitializeLCD();

InitializeKeypad();

// Display welcome message

LCD_SendByte(0x01, 0);  // Clear display

for(volatile int i = 0; i < 200000; i++); // Clear delay

// Display "Hello World!"

char* message = "Hello World!";

while(*message) {

LCD_SendByte(*message, 1);  // Send character

message++;

}

// Position cursor on line 2

LCD_SendByte(0xC0, 0);  // Set cursor to line 2, position 0

// Display "Key: "

char* prompt = "Key: ";

while(*prompt) {

LCD_SendByte(*prompt, 1);

prompt++;

}

// Main loop

while(1) {

char key = ReadKeypad();

if(key != 0) {

// Display the pressed key

LCD_SendByte(0xC5, 0);  // Position after "Key: "

LCD_SendByte(key, 1);   // Display the key

LCD_SendByte(' ', 1);   // Space after key

// Clear screen if '*' is pressed

if(key == '*') {

LCD_SendByte(0x01, 0);  // Clear display

for(volatile int i = 0; i < 200000; i++);

LCD_SendByte(0xC0, 0);  // Line 2

char* newprompt = "Key: ";

while(*newprompt) {

LCD_SendByte(*newprompt, 1);

newprompt++;

}

}

SysTick_Debounce();  // Debounce delay

}

// Small scan delay

for(volatile int i = 0; i < 50000; i++);

}

}